- Packets may be in transit during the switchover in networks.  How many packets are lost?

- Notation:

  - tMNFAi = delay from mobile node to i-th FA

  - tHAFAi = delay from HA to i-th FA

  - tCNHA = delay from CN to HA

  - dDATA = duration of data packet

- Assumptions: control packets have negligible duration; no delay for computation; no errors.

- Based on the figure, the total delay between the link breaking and being re-established is

(Eq. 1)

- Two options: retransmit or buffer

  - Retransmit: all lost packets are dropped and retransmitted by the CN.  Advantage: simple; disadvantages: wastes bandwidth, larger latency.

  - Buffer: a required number of packets are retained by the HA in a buffer in case the foreign network changes.  Advantages: low latency; disadvantages: complexity at the HA.

- (Example.)

- Aside from packet loss, out-of-order packet delivery is a problem, whether you buffer or retransmit (though it is better with buffering)

- Soft handover: Maintain simultaneous links to two foreign networks during the handover process to ensure that packets are not lost – also helps with out-of-order delivery

- Disadvantage: Wasteful of bandwidth, though usually for only a short time

(Fig. 1)

Micro-mobility

- Say you are designing a cell network.  As a result of the above analysis, you want to minimize packet losses.

- Travelling between adjacent cells, owned by the same company, it seems excessive to rebuild a link from scratch after every handoff

- Cellular IP:

  - Group several base stations into a cluster, and assign a common FA to the entire cluster

  - This common FA is called the Cellular IP gateway

  - Downstream of the CIP gw is a router to redirect packets in case of handoff within the cluster

- Thus, packet losses only occur when travelling between clusters, which is less frequent than travelling between base stations

(Fig. 2)

- packets are essentially never lost, and the smaller delay among the localized nodes means that fewer out-of-order packets arrive

Mobility support in IPv6

- First cellular phones: 1973 (prototypes), 1978 (first commercial network in Chicago)

- First portable computers: Osborne 1 (1981), Compaq Portable (1983 – 28 lb)

- Internet Protocol: 1973 (proposal), 1981 (IPv4)

- So it’s reasonable that no mobility support would be built in to IPv4.  However, IPv6 (1998) is designed with device mobility in mind.

- Fundamentally, mobility in IPv6 is similar to Mobile IP as an extension of IPv4.  However … changes in IPv6:

  - All IPv6 nodes must do address autoconfiguration – so it’s easy for nodes to acquire a COA.

  - All IPv6 nodes can send address bindings to any other node – so the MN can send its new COA directly to the HA.

  - As a result of the above two facts, FAs are no longer needed.

  - Soft handover support: the MN can now send address bindings directly to the router handling old versions of the COA (as a result packet loss is minimized)

  - Address space massively increased, so it is not “wasteful” for every mobile device to keep two IP addresses