Applications: Cellular Telephony and Data Services

- We now switch gears to talk about practical applications

- First up: A review of the basic concepts of cellular telephony

- Space is partitioned into regular regions known as “cells” (hence the name “cellular”).

- Each cell is served  by a “base station”

- While within a cell, a mobile handset will create a radio link to that cell’s base station

(Fig. 1)

- When mobiles move from one cell to another, a “handover” occurs – ensures calls and/or data sessions are not dropped.

- Why cellular? – Spatial reuse of frequencies.

- Two base stations sufficiently far apart will not interfere with each other – thus it is safe for them to use the same frequencies (i.e., the frequencies can be re-used)

- As long as cells between the re-using cells use different frequencies, there is no interference

- This leads to “re-use factors” and “re-use patterns”

(Fig. 2)

- Example.  (a) A cellular phone system uses FDMA.  The total available bandwidth is 98 MHz, and each individual call occupies 20 kHz.  The re-use factor is 7.  How many calls can be made simultaneously per cell?

- Example. (b) Same system.  Each cell occupies 0.5 km2.  The area of the city is 28 km2.  How many calls can be made simultaneously across the city?

- Aside.  Area of a hexagon.

(Eq. 1)

- so a cell occupying 0.5 km2 has a radius of 438.7 m

- How would you increase the number of calls in the city?

  - Shrink the cell size (disadvantage: cost); and/or

  - Reduce the re-use factor (disadvantage: interference).

  - WRONG ANSWERS: Increase bandwidth (usually not possible) or decrease bandwidth per call (would decrease the quality of service).

- Example. (c) Same system as in example (a) and (b).  Each cell occupies 0.25 km2.  How many simultaneous calls?

- Example. (d) Same system.  Re-use factor is now 4.  How many simultaneous calls?

Cellular “generations” and common protocols

- First generation (1G): Analog telephone service

  - 1980s-1990s; usually FDMA

  - Advanced Mobile Phone System (AMPS)

- Second generation (2G): Digital telephone and limited data service

  - 1991-present.  Reduced power requirements; Vocoders reduced the required bandwidth; security.

  - GSM: FDMA+TDMA-based, worldwide

  - IS-95: CDMA-based, N. America and Asia

- “2.5G”: Packet-switched data service (GPRS) at low speeds (56-114 kbps); EDGE (higher speed).

- Third generation (3G): High-speed data services

  - 2001-present. 348 kbps-14.4 Mbps

  - IMT-2000 is the internationally agreed standard, with several sub-standards

- Fourth generation (4G)

  - Expected commercial availability in 2010

  - 100 Mbps to 1 Gbps in the downlink

  - Applications such as HDTV to the mobile