Wireless Spectrum Continued

| Name | Band | Application |
| --- | --- | --- |
| ELF, SLF, VLF LF | Up to 30 kHz 30—300 kHz | Military, navigation |
| MF | 300 KHz—3 MHz | AM radio |
| HF | 3—30 MHz | Shortwave radio, amateur radio |
| VHF | 30—300 MHz | FM radio, TV, point-to-point comm |
| UHF | 300 MHz—3 GHz | TV, cellphones, ISM bands |
| SHF EHF | 3—30 GHz 30—300 GHz | ISM, Satellites, high-capacity links |

- Each slice of the wireless spectrum is licensed for a specific task, and by law cannot be used for any other purpose

- E.g. radio stations are licensed to the station using them by the government, and by law can only be used by that station for approved purposes

- An exception is the “ISM band” (ISM = Industrial, Scientific, Medical) – these are unlicensed bands which may be used for any purpose (subject to power restrictions)

- There are several in various parts of the wireless spectrum, but the most popular is from 2.4-2.5 GHz; used for WiFi, Bluetooth, Zigbee, etc.

Path Loss

- Wireless signals decay with distance from the transmitter

- In free space, the decay is proportional to the size of the wave front (conservation of energy)

(Fig. 1)

- in empty three-dimensional space, the surface area of a sphere is proportional to r2 (more precisely it’s 4r2), so this is how fast signals decay

- If signal decay is proportional to rd, then d is called the path loss exponent

- Real-world signals can bounce off the ground, buildings, etc., so in practice d can be between 2 and 4

- On the other hand, if the signal is guided (e.g. by a tunnel), d can be less than 2 (doesn’t happen often).

(Fig. 2)

- example: 1 W signal, 100m from tx to rx, if rx power is 0.1 W with d=2, assuming same constant of proportionality, what is rx power with d=3 and d=4?

- Answer: 1 W, 0.01 W

More on signal propagation

- Main effects on radio signals other than attenuation: scattering, shadowing, diffraction, reflection

(Fig. 3)

Antenna Design

- Antennas have an antenna pattern, the relative signal strength (either transmit or receive) from all directions

- Some examples: isotropic, dipole, sector

(Fig. 4)

- A directional antenna is designed by creating physical reflectors/blockers (e.g., dish) or by combining many antenna elements (e.g., array)

- Project signal where wanted, reject interference from unwanted sources, eliminate extra paths

The Mobile Radio Environment: Multipath Fading

- As mentioned, wireless signals can follow many paths from transmitter to receiver (reflection, scattering, etc.)

- These signals have different lengths and therefore different phases

(Fig. 5)

- For a sinusoid with frequency fc, phase at the rx is

(Eq. 1)

- Say you have two paths, one with distance d1, the other with distance d2.  They combine at the receiver to form

(Eq. 2)

- Trig identity: sin a + sin b = 2 sin((a+b)/2) cos((a-b)/2)

(Eq. 3)

- since sin is always between -1 and 1, the effect of the fading is to diminish the signal amplitude, possibly to nothing!