- Example.  d1-d2 = 1.5m, f=100 MHz

Power vs. Amplitude

- Amplitude refers to the field strength of an E or M field

- Power is proportional to the square of the field strength

- So power decaying proportional to r2 (path loss exponent) means field strength decays proportional to r

- Example.  Received amplitude as given last class (Eq. 3) has power

(Eq. 1)

Plane Earth Path Loss

- As a practical example of two-ray multipath fading, consider a signal propagating over a flat plane

(Fig. 1)

- hT is tx height, hR is rx height, d is the distance from tx to rx, R is the reflection coefficient (i.e. coefficient multiplied by reflection)

- received signal is

(Eq. 2)

- If d is sufficiently large, R = -1 (from physics)

- Trig identity: sin a – sin b = 2 cos((a+b)/2) sin((a-b)/2)

(Eq. 3)

- d2 – d1 is … using the approximation (1+u)1/2  1 + u/2

(Fig. 2)

(Eq. 4)

- furthermore sin u  u for small u

(Eq. 5)

- so the path loss exponent for plane earth propagation is 4.

- Limitations: Earth needs to be very flat and a very good conductor in order for this to hold (e.g., calm ocean)

- On dry ground, path loss exponent is close to 2 as long as line-of-sight path exists from tx to rx

Rayleigh fading

- What about lots and lots of sinusoids?

- Remember that sin(a+b) = sin a cos b + cos a sin b, so

(Eq. 6)

- The A cos  and A sin  terms are random (because path length is random, and phase is random)

- Mean zero, variance small (but the same for both)

- So in the end we get

(Eq. 7)

- Thanks to the central limit theorem, sums of large numbers of random variables approach the Gaussian distribution, so U and V are Gaussian random variables

- Another trig identity: U cos a + V sin a = (U2+V2)1/2sin(a+b), where

(Eq. 8)

- The signal strength is then (U2+V2)1/2, which has the Rayleigh distribution, which has CDF

(Eq. 9)

- The extra phase doesn’t matter – can be tracked

- r2 is the average power of the sinusoid (given)

- tells us the probability that the signal strength will be less than a given value in an environment with lots of paths

- Example: Average power is 1 W.  What is the probability that the signal strength falls below 1?

Flat vs. frequency-selective fading

- How different is fading from frequency to frequency?

- Recall Eq. 3

(Eq. 10)

- if d1-d2 is large, a small change in frequency will have a huge effect (frequency-selective fading)

- on the other hand if d1-d2 is small, a small change in frequency will have a negligible effect (flat fading)

Delay Spread and Coherence Bandwidth