WiFi Medium access control

- Basic principle: Carrier-sense multiple access with collision avoidance (CSMA/CA)

- Similarly to Ethernet, nodes sense the medium and only transmit when it is idle

  - Once the medium is idle, everyone waits an additional period of time (Distributed Coordination Function Inter-Frame Spacing, DIFS) before transmitting – this allows high-priority messages to get through

  - After DIFS has expired, each node selects a random backoff time and continues to wait

  - If at any point up to the expiry of the backoff timer the medium is busy, the node must start over

  - Otherwise, once the backoff timer expires, the node may transmit

(Fig. 1)

- This is possibly unfair if a node repeatedly selects a large backoff time – that node won’t be able to transmit

  - If a backoff timer is interrupted by a transmission, the node keeps the old value of the backoff timer and continues from where it left off

  - Maximum idle time is thus limited to the original value of the backoff timer

  - Example

- Shorter delays are possible for high priority messages

  - Short inter-frame spacing (SIFS) – for control messages, e.g., packet acknowledgments

  - Point coordination function inter-frame spacing (PIFS) – for time-bounded services.

(Fig. 2)

Wireless Sensor Networks

- By now, two-way digital radios are inexpensive, as is computing power

- In many applications, the more data collected (from as close as possible to the phenomenon), the better

  - Example. Wildlife tracking.

  - Conventional method: Radio transponder collars.

  - New method: Sensor networking collars that continuously monitor themselves and their neighbors

(Fig. 3)

- Sensor network features

  - Inexpensive – many devices can be used, okay to lose them, add more as needed

  - Robust – Tolerant of device failure/addition

  - Distributed – auto-configuration, no single master or point of failure

  - Ubiquitous – measurements taken from as many locations as possible, as close to the phenomenon as possible

- Sensor network challenges

  - Power management – how to optimize devices for extremely long life?

  - Low complexity – how to deal with devices that have reduced computing requirements?

  - Organization – how to perform network control in a distributed manner?

  - Routing – how to get data from one place to another in a dynamic network?

IEEE 802.15.4

- IEEE 802.15.4 is a broad wireless networking standard that addresses some of the challenges in sensor networking

- As in Bluetooth and WiFi, only the bottom two layers are specified – certain protocols, e.g. ZigBee, are extensions of 802.15.4 into higher layers

- 802.15.4 physical layer:

  - ISM bands at 2.4 GHz (worldwide); other channels in North America and Europe

  - Data rates ranging from 20-250 kbps

- 802.15.4 MAC layer:

  - Direct sequence spread spectrum; slots and/or CSMA/CA (same as WiFi).

- 802.15.4 architecture:

  - Unlike Bluetooth, distinguishes between “reduced function devices” (RFD) and “full function devices” (FFD)

  - Only FFDs can be “coordinators” (like masters in Bluetooth); RFDs only connect to FFDs