IEEE 802.15.4

- 802.15.4 architecture:

  - Unlike Bluetooth, distinguishes between “reduced function devices” (RFD) and “full function devices” (FFD)

  - Only FFDs can be “coordinators” (like masters in Bluetooth); RFDs only connect to FFDs

  - Star and peer-to-peer topologies are possible

  - Star topologies can be organized into a hierarchy

    - Network coordinator (global)

    - PAN coordinator (local star cluster)

    - Non-coordinator FFD and RFD (leaves)

(Fig. 1)

Beacons and Superframes

- IEEE 802.15.4 permits the organization of time into “superframes”

- Coordinators send “beacon” packets to organize the superframes (for synchronization and control)

- A superframe consists of:

  - Contention access period (CAP) – e.g., access using CSMA

  - Contention free period (CFP) – guaranteed slots for certain devices, assigned by coordinator

  - Inactive period

  - Beacon interval and superframe duration specified by coordinator in the beacon packet

  - Why have an inactive period? – to allow devices to sleep.

(Fig. 2)

Routing in sensor networks: AODV

- AODV = Ad hoc On demand Distance Vector routing

- One of the routing techniques allowed in ZigBee

- Features:

  - Strictly on-demand: routes don’t keep routing tables unless in an active route; routes are only formed when needed

  - Avoid stale routes: network is dynamic so all routes must be maintained “fresh”

  - Local and distributed: global coordination and routing not needed

- Path Discovery

  - When a node needs a route to the destination, it transmits a Route Request (RREQ) to its neighbors

  - RREQ consists of: source address, source sequence #, broadcast ID, destination address, destination sequence #, hop count

  - Source address and broadcast ID identify the RREQ

  - Source sequence # maintains “freshness” of reverse route to the source

  - Destination sequence # is the last known sequence # used by the destination – source won’t accept any route with a more “stale” sequence number

  - If the neighbor has a route to the destination, it informs the source

  - If no route to the destination, neighbor broadcasts the RREQ to its neighbors & increments hop count

- Reverse Path

  - RREQ travels from the source to various possible intermediate routes – sets up an automatic reverse path

  - Reverse path entries are maintained by each node for at least enough time for the route information to propagate back to the source

- Forward path setup

  - Eventually a path is found – either the RREQ arrives at the destination, or a node with a route to the destination

  - Freshness: If not the destination, the destination sequence #s are compared – if the node’s dsn is smaller than the source’s dsn, this is a “stale” route and the RREQ is forwarded again

  - Otherwise the node returns a route reply (RREP) to the neighbor from which it received the RREQ

  - RREP propagates back towards the source, and the route is set up

  - Other nodes drop the routing information after a timeout has expired

(Fig. 3)