# York University

Dept. of Computer Science and Engineering

EECS4201

Computer Architecture

Quiz 3

Dec 4, 2014

Time allowed: 20 minutes

Name ______________________                                            ID_____________

# Question 1 – 5 points

Consider the following code

LD	F2, 32(R4)

ADD	F2, F2, F6

MUL	F6, F2, F2

ADD	F4, F8, F8

ADD	F4, F8, F8

Assume a CPU that uses Tomasulo’s algorithm. We have one integer ALY, one load/store unit, and 2 Floating point units. Assume that we have an infinite number of reservation station per unit (that is of course not practical, but easier for a 20 minute quiz).

At what time each instruction will be issued, starts execution, finishes execution, and write back the result. Assume the following time for calculation

FP ADD 4 cycles		FP MUL 10 cycles

Integer add 1 cycle		memory 1 cycle

| Instruction | fetch | start EX | Ends EX | WB |
| --- | --- | --- | --- | --- |
| LD F2, 32(R4) | 1 | 2 | 3 | 4 |
| ADD F2, F2, F6 | 2 | 5 | 8 | 9 |
| MUL F6, F2, F2 | 3 | 10 | 19 | 20 |
| ADD F4, F8, F8 | 4 | 5 | 8 | 9 |
| ADD F4, F8, F8 | 5 | 10 | 13 | 14 |

Can start since only 1 FP unit is busy (ADD F2)

Must wait till second ADD finishes since we have only 2 FP units